# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: CNC-WOOD-BASE-01

Title: 2D CNC Milling Exercise: Pocket and Profile Cutting in Wood-Based Panels

Date of submission: may 2015

Source (URL): https://cdn.dal.ca/content/dam/dalhousie/pdf/faculty/architecture-planning/school-of-architecture/technical-skills/2d-cnc-exercise.pdf

Original author (if applicable): E.Jannasch, N. Merrithew, E. Wilson

License (mandatory):

License link:

Language: English

Keywords:

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): CNC
 Level (Basic / Intermediate / Advanced): Basic
 EQF level: 3–4
 Estimated duration: 4–6 hours
 Target learners:

- VET students in woodworking and furniture production

- Beginners in CNC machining

- Learners in technical training pathways (digital fabrication)

## 3. LEARNING OBJECTIVES

The goal of the exercise is to introduce learners to fundamental CNC machining operations through a structured 2D milling task.

Students will learn how to perform pocketing and profile cutting operations, understanding the relationship between digital design, toolpaths, and physical output.

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

- Understand basic CNC workflow (CAD–CAM–machine)

- Execute pocketing (recess) operations

- Perform profile cutting operations

- Set up CNC machining parameters

- Identify and correct machining issues

- Evaluate the quality of milled components

## 5. COMPETENCES

Knowledge:

- Basic principles of CNC machining

- Types of CNC operations (pocketing, profiling)

- Properties of wood-based materials (e.g. MDF)

- CAM concepts (toolpaths, depth, feeds and speeds)

Skills:

- Prepare a file for CNC machining

- Generate basic toolpaths

- Set up CNC machine (zeroing, material fixing)

- Execute machining operations

- Inspect and evaluate output quality

Responsibility & autonomy:

- Operate CNC equipment safely under supervision

- Follow workshop safety procedures

- Perform basic troubleshooting independently

## 6. EXERCISE DESCRIPTION

The exercise consists of producing a simple machined panel using two fundamental CNC operations:

- Pocketing (recess milling): creating a shallow cavity in the material

- Profile cutting: cutting the external contour of the piece

Learners will:

- Prepare a 2D design file

- Define machining parameters

- Generate toolpaths

- Perform CNC milling on a wood-based panel (e.g. MDF)

The exercise focuses on understanding how different operations affect the final result and how to control machining precision.

## 7. TECHNICAL INFORMATION

CNC milling machine (3-axis router)

Wood-based panels (MDF recommended)

CAD/CAM software (e.g. Fusion 360, RhinoCAM, VCarve)

End mill (e.g. 6 mm)

Clamping system or vacuum table

Measuring tools

PPE (safety glasses, hearing protection)

## 8. IMPLEMENTATION- STEP BY STEP

How can this model be used in the classroom?
 The exercise can be implemented as an introductory CNC workshop where all students perform the same machining operations to understand the basics of CNC milling.

What methodology can be used for its integration into the classroom/curriculum?
 The exercise follows a guided hands-on approach:

- Introduction to CNC and safety procedures

- Explanation of pocketing and profile cutting operations

- Preparation of design file

- CAM setup and toolpath generation

- Machine setup (zeroing, material fixing)

- Execution of pocketing operation

- Execution of profile cutting

- Evaluation of the final piece

- Discussion of errors and improvements

What benefits can be obtained with its use?

- Clear understanding of basic CNC operations

- Strong connection between theory and practice

- Immediate visual and physical feedback

- Development of technical precision

- Foundation for more advanced CNC tasks

## 9. EXPECTED RESULTS

A machined panel with a correctly milled pocket and external profile

Understanding of basic CNC operations

Ability to identify machining errors

## 10. EVALUATION CRITERIA

Criterion 1: Correct setup and safe use of CNC machine
 Criterion 2: Accuracy of pocket depth and profile dimensions
 Criterion 3: Surface quality and overall machining precision

Assessment method (rubric/checklist/ peer review, etc.): Checklist + practical assessment
 Type (formative/summative): Formative and summative

## 11. COMMON ERRORS / TROUBLESHOOTING

Error 1:

Error 2:

## 12. FILES PROVIDED

File 1:

File 2:

...

## 13. ADAPTATION & CONTEXT

Original context of the exercise:
 Architecture and digital fabrication training in higher education

Adaptation made for woodworking VET:
 Use of wood-based materials (MDF) and focus on CNC skills for furniture production

Suggested course/module:

- Introduction to CNC woodworking

- Digital fabrication basics

Prerequisites:

- Basic ICT skills

- Basic understanding of workshop safety

## 14. TRANSFERABILITY

Applicability to other VET sectors:

- Metal machining

- Plastics processing

- Product prototyping

Required adaptations:

- Material-specific machining parameters

- Tool selection adjustments

## 15. ADDITIONAL NOTES

This exercise provides a foundational introduction to CNC machining and is particularly suitable as a first step before more complex operations such as joinery, nesting, or multi-component furniture production.

It aligns with Industry 4.0 learning approaches by integrating digital design and automated manufacturing processes.