# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise8

Title: 3D Design and Printing of a Dowel Drilling Template

Date of submission: 27/04/2026

Source (direct URL to the exercise):

Original author (if applicable): IES JOSE LUIS CASTILLO PUCHE (JLCP)

License (e.g. CC BY, CC BY-SA, etc.): Open license (Creative Commons type, according to MakerWorld platform)

Language: English

Keywords: 3D printing, woodworking, dowels, templates, digital fabrication, wood, precision

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D Printing

Level (Basic / Intermediate / Advanced): Intermediate

EQF level: 4 (Intermediate Vocational Training)

Estimated duration: 4–6 hours

Target learners:
VET students in Intermediate Level Vocational Training (Woodworking and Furniture Production)

## 3. LEARNING OBJECTIVES

Objective 1: Understand the use of 3D printing applied to woodworking tools
Objective 2: Download, prepare, and adapt 3D models for manufacturing
Objective 3: Improve accuracy in joints using drilling templates

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

LO1: Download and analyze functional 3D models from open repositories
LO2: Prepare files for 3D printing using open-source slicing software
LO3: Use 3D printed templates to perform accurate drilling in wood

## 5. COMPETENCES

Knowledge: - Understanding of FDM (Fused Deposition Modeling) 3D printing technology and its direct application in the woodworking and furniture industry.

- Knowledge of 3D printing slicing parameters (infill density, wall thickness, patterns) necessary to achieve structural integrity and mechanical resistance.

- Understanding of traditional dowel joinery techniques, tolerances, and the importance of precision tooling for flush assemblies.

Skills: - Ability to prepare and manipulate 3D models using open-source slicing software (importing, rotating, slicing, and exporting G-code).

- Competence in setting up, calibrating, and operating an FDM 3D printer safely.

- Dexterity and precision in using portable electric tools (drills) and auxiliary jigs to machine and assemble wooden components.

Responsibility & autonomy: - Working safely and independently following occupational health and safety regulations in both the computer lab and the woodworking workshop.

- Taking responsibility for the proper use and monitoring of digital fabrication equipment (3D printers) and hand/power tools.

- Autonomy in troubleshooting basic 3D printing issues (e.g., bed adhesion) and critically assessing the final quality and precision of the woodworking joint.

## 6. EXERCISE DESCRIPTION

Students will work with a 3D model of a dowel drilling template downloaded from an open platform. They will learn how to prepare the file for 3D printing using open-source software, adjusting key parameters such as scale, infill, and supports. Once printed, the template will be used in the woodworking workshop to perform accurate drilling on wooden pieces, evaluating its functionality. This activity connects digital fabrication with traditional woodworking processes, improving precision while promoting the use of open technologies.

## 7. TECHNICAL INFORMATION

Required equipment:

- FDM 3D printer

- Computer

- Drill press or handheld drill

Software needed:

- FreeCAD (modeling and visualization)

- Ultimaker Cura (open-source slicing software)

Materials:

- PLA filament

- Wooden workpieces for testing

File formats included:

- STL

## 8. IMPLEMENTATION- STEP BY STEP

Group size: 2–3 students

Teacher role:
Guide the process, provide technical support, and supervise the use of software and equipment

Teacher preparation required:

- Review the 3D model

- Prepare the 3D printer

- Ensure software is installed and functional

Set up time: 30 minutes

Steps:

- Access the link and download the STL file

- Open the file in slicing software (Cura)

- Configure printing parameters (layer height, infill, supports)

- Print the template using the 3D printer

- Use the printed template in the workshop to drill holes in wood

- Evaluate accuracy and functionality

## 9. EXPECTED RESULTS

- Functional drilling template produced

- Improved drilling accuracy

- Understanding of the 3D printing workflow

- Integration of digital tools into woodworking processes

## 10. EVALUATION CRITERIA

Criterion 1: Correct preparation of the file for printing
Criterion 2: Quality of the printed part
Criterion 3: Proper use of the template in woodworking

Assessment method (rubric/checklist/ peer review, etc.): Rubric

Type (formative/summative): Formative

## 11. COMMON ERRORS / TROUBLESHOOTING

Error 1: Poor bed adhesion → Solution: adjust temperature and bed leveling

Error 2: Incorrect dimensions → Solution: check scaling and slicer settings

## 12. FILES PROVIDED

File 1: Downloaded STL file
File 2: Cura printing profile/settings

## 13. ADAPTATION & CONTEXT

Original context of the exercise:
General use of 3D printing for tool creation

Adaptation made for woodworking VET:
Direct application to dowel drilling processes in woodworking joints

## 14. TRANSFERABILITY

Can this be reused without modification? (Yes/No) NO

If not, what needs to change?:

Adaptation to available equipment and student skill level

## 15. ADDITIONAL NOTES

Highly motivating activity that connects emerging technologies with traditional craftsmanship. It promotes hands-on learning and the use of open-source resources.