# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise9

Title: SCAN_ADV_02_Column - 3D Scanning and Digital Archiving of Ornate Architectural Elements

Date of submission: 24/04/2026

Source: https://www.artec3d.com/3d-models/golden-column

Original author: Artec 3D Model Library

License: Creative Commons (Educational Use)

Language: English

Keywords: #3DScanning #Architecture #Restoration #DigitalTwin #GoldLeaf #OrnateDesign

## 2. CLASSIFICATION

Technology: 3D Scanning

Level: Advanced

EQF level: 5

Estimated duration: Scanning: 1.5h / Data Processing: 4h

Target learners: Restoration specialists, Architecture students, Woodworking technicians.

## 3. LEARNING OBJECTIVES

Objective 1: Master the scanning of reflective or metallic surfaces (gold leaf) which are traditionally difficult for optical scanners.

Objective 2: Learn to combine multiple scan angles to capture deep recesses in ornate carvings.

Objective 3: Process high-resolution texture maps to preserve the visual integrity of historical artifacts.

## 4. LEARNING OUTCOMES

LO1: Learners will be able to set up optimal lighting and scanner settings for reflective surfaces.

LO2: Learners will be able to create a watertight 3D mesh from a complex architectural column.

LO3: Learners will be able to export high-fidelity models for virtual reality (VR) or CNC reproduction.

## 5. EXERCISE DESCRIPTION

This exercise involves the digital "preservation" of a decorative golden column. The challenge lies in the intricate carvings and the reflective nature of the gold finish. Students must navigate the scanner around the column to ensure no "shadow areas" are left in the digital mesh. This is a typical task in heritage restoration where a physical replica might be needed.

## 6. TECHNICAL INFORMATION

Required equipment: High-precision handheld 3D scanner (e.g., Artec Space Spider or Eva).

Software needed: Artec Studio or specialized mesh processing software (MeshLab/Autodesk).

Materials: Physical ornate column (or the provided digital sample for processing practice).

File formats included: .OBJ (with high-res texture), .STL, .WRL.

## 7. IMPLEMENTATION- STEP BY STEP

Group size: 2 students.

Teacher role: Technical supervisor.

Steps: 1. Preparation: If scanning a real gold surface, determine if scanning spray or specialized polarizing filters are needed. 2. Scan Paths: Move the scanner in vertical paths, overlapping each pass by 60%. 3. Base Capture: Ensure the pedestal and the top joint are captured for proper orientation. 4. Alignment: Use software to "stitch" the vertical scans into a single 360-degree model. 5. Texture Mapping: Apply the photographic data over the mesh to recreate the golden luster.

## 8. EXPECTED RESULTS

A high-poly digital twin of the column that shows every detail of the carving and the texture of the gold leaf.

## 9. EVALUATION CRITERIA

Criterion 1: Mesh density and detail in the carved sections.

Criterion 2: Texture alignment (no "ghosting" or blurred areas).

Criterion 3: Accurate scale compared to the original piece.

Type: Summative.

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: "Holes" in deep carvings. Solution: Tilt the scanner at a 45-degree angle to "see" into the recesses.

Error 2: Inaccurate geometry due to reflections. Solution: Reduce ambient light or use a scanner with better texture-tracking capabilities.

## 11. FILES PROVIDED

File 1: Golden_Column_Interactive_Model (Direct link to the 3D viewer and data).

## 12. ADAPTATION & CONTEXT

Context: Adapted from professional museum archiving techniques for VET training in furniture and architectural restoration.

## 13. TRANSFERABILITY

Other courses: Interior Design, History of Art, Digital Manufacturing.

## 14. ADDITIONAL NOTES

This specific model is an excellent example of "Reverse Engineering" where a digital file is used to create a mold for casting or a program for 5-axis CNC carving.