# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise7

Title: 3D Scanning of Classic Ornaments and Their Integration into Open CNC Designs

Date of submission: 27 April 2026

Source (URL): https://www.cncmasters.com/17-cnc-furniture-design-resources/

Original author (if applicable): Wood and Furniture VET Teaching Staff (Erasmus Project)

License (mandatory): CC BY-SA 4.0 (Creative Commons Attribution-ShareAlike 4.0)

License link: https://creativecommons.org/licenses/by-sa/4.0/

Language: English

Keywords: 3D Scanner, Geometric Mesh, STL, Reverse Engineering, CNC Furniture, Free Software, Wood, Open Source Repositories.

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D Scanning & CNC Integration

Level (Basic / Intermediate / Advanced): Intermediate

EQF level: 4 (Corresponding to Intermediate Vocational Training)

Estimated duration: 4 hours (2h scanning workshop practical sessions + 2h CAD computer room)

Target learners: Intermediate Level vocational Training (Woodworking and furniture Production)

## 3. LEARNING OBJECTIVES

Objective 1: Understand and execute the technical process of 3D scanning (geometric capture) of a real wooden element or workshop ornament.

Objective 2: Clean, orient, and optimise a 3D mesh obtained from scanning using open-source software.

Objective 3: Modify and scale the scanned part to successfully integrate it into an open-source CNC furniture design obtained from web repositories.

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

LO1: Prepare a real wooden part and operate it under 3D scanner software (or photogrammetry) to obtain a dense and reliable point cloud.

LO2: Repair STL or OBJ files, filling mesh holes and reducing polygons to ensure they are watertight and suitable for subsequent machining or 3D printing.

LO3: Navigate online furniture design libraries (based on the reference link resources) to download a base model and combine the digitised physical part with the parametric design.

## 5. COMPETENCES

Knowledge: Physical and optical principles of 3D digitisation. Typology of digital manufacturing files (.STL, .OBJ, .DXF). Search methodologies in web repositories.

Skills: Dexterity in the physical handling of scanning tools. Operation of 3D mesh editing software. Use of callipers and tape measures for comparative dimensional control in the digital space.

Responsibility & autonomy: Autonomous planning of the workflow, from capture to final file. Ability to independently troubleshoot scanning errors.

## 6. EXERCISE DESCRIPTION

Starting from the reading of the resources provided in the CNC Masters web article, students will explore open furniture design repositories (such as Thingiverse, Yeggi, or 3axis.co). Each team will download the plans for a simple functional piece of furniture (for example, a CNC side table). Subsequently, they will apply "Reverse Engineering": they will choose a physical workshop ornament, such as a classic carved leg or a complex moulding, and digitise it using a 3D Scanner. Once the mesh of the piece is obtained, they will use free software (MeshLab) to clean it and assemble it virtually (FreeCAD / Blender), replacing a leg or handle of the downloaded model. The result is a hybrid piece of furniture designed to be manufactured using numerical control.

## 7. TECHNICAL INFORMATION

Required equipment: 3D scanner (or smartphone/DSLR camera for use with photogrammetry software), high-performance computer for graphic processing, vernier calliper.

Software needed: * Meshroom (free license - if photogrammetry is used instead of a laser scanner) or native scanner software.

- MeshLab (free license - for mesh editing, cleaning, and curing).

- FreeCAD or Blender (free license - to assemble the scanned part with the CNC furniture).

Materials: Ornamental wooden piece (Louis XV leg, corbel, carved handle), mattifying spray / talcum powder (if the piece is glossy), scanning targets (markers).

File formats included: .STL (Scanned model), .DXF or .STEP (Downloaded CNC plans).

## 8. IMPLEMENTATION- STEP BY STEP

Group size: Pairs (2 students).

Teacher role: Technological facilitator. The teacher performs a prior demonstration of the scanner's use and guides the students in using online open design repositories.

Teacher preparation required: Pre-select real workshop pieces with interesting geometry. Install MeshLab and FreeCAD on the computer room equipment. Review the CNC Masters link to confirm which websites are optimal for that day.

Set up time: 20 minutes (Preparation of the scanning area and booting up the computers).

Steps:

- Research and Download: Students access the furniture resources article, log into platforms like Yeggi or Thingiverse, and download a basic CNC furniture design.

- 3D Scanning (Capture): They prepare the physical wooden piece, place marks/targets on it if necessary, and perform the sweep with the 3D scanner until the base mesh is obtained. Real measurements of the piece are taken with the calliper.

- Mesh Curing (MeshLab): They import the raw 3D model, remove noise, cut unnecessary bases, fill holes ("fill holes"), and export it as a solid .STL file.

- Assembly and Scaling (FreeCAD/Blender): They import the downloaded CNC model and the scanner's STL file. They scale the scanned piece, checking the real dimensions taken with the calliper, and visually integrate it into the furniture assembly.

## 9. EXPECTED RESULTS

An assembled digital CAD file (for example, in .FCStd format or a 3D PDF) that demonstrates the perfect integration of a 3D-scanned classic wooden piece into the chassis of a modern open-source CNC furniture piece, respecting scales and proportions.

## 10. EVALUATION CRITERIA

Criterion 1: Digitisation quality: The final STL file resulting from the scan is clean, is a closed (watertight) mesh, and has no inverted polygons or "noise" around the piece.

Criterion 2: Dimensional accuracy and scale: The dimensions in the CAD software of the scanned piece correspond faithfully (tolerance of +/- 1 mm) to the original physical piece in the workshop.

Criterion 3: Digital integration and Autonomy: The student has correctly used the indicated repositories to find the CNC model and has successfully modified it, demonstrating technical independence in solving IT problems.

Assessment method (rubric/checklist/ peer review, etc.): Technical rubric of the digital file (teacher assessment) combined with a completed workflow Checklist.

Type (formative/summative): Formative assessment (during the workshop process) and summative assessment (final grading of the assembled CAD file).

## 11. COMMON ERRORS / TROUBLESHOOTING

Error 1: The scanner loses geometry tracking ("Tracking lost") during capture. Solution: Add more marks (scanning targets) on the wooden piece, improve the workshop lighting avoiding direct sunlight, or apply a mattifying spray if the piece has high-gloss varnishes.

Error 2: Scale mismatch when importing the STL into FreeCAD. Solution: Students often forget that STL formats are dimensionless (they do not save the unit of measurement by default). They must use the global scaling tool using the physical measurement taken with the calliper in step 2 as a reference.

## 12. FILES PROVIDED

File 1: 3D_Scanning_and_MeshLab_VET_Manual.pdf (Quick guide prepared by the teacher on how to clean point clouds).

File 2: Open_Furniture_Repositories_List.md (Document with filtered links from the main CNC Masters article suitable

## 13. ADAPTATION & CONTEXT

Original context of the exercise: Search and optimisation of ready-to-cut CNC designs based on global online repositories for makers and cabinetmakers.

Adaptation made for woodworking VET: Reverse Engineering technology (3D Scanning) is introduced. Trainee carpenters will not only manufacture parts from the web but will learn to capture historical replacement parts (e.g., restoration pieces, altarpiece cornices, or Elizabethan legs) to combine them with modern CNC manufacturing methodologies.

Suggested course/module: "Carpentry and Furniture Projects" or "Machining of Wood and Derivatives" modules.

Prerequisites: Very basic knowledge of computer-aided design (2D/3D CAD), understanding of scales (1:1), and graphic interpretation of views and isometrics.

## 14. TRANSFERABILITY

Applicability to other VET sectors:

Required adaptations:

## 15. ADDITIONAL NOTES