# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: NewDevelopments_Exercise3

Title: CNC programming for machining a drawer front

Date of submission: 21/04/2026

Original author (if applicable): José Antonio Ibáñez Palao

License (e.g. CC BY, CC BY-SA, etc.): Creative Commons Attribution-NonCommercial-Share Alike 4.0 International License

Language: Spanish

Keywords: CNC, 3d machining, 3axis, roughing/finishing operation, pocketing operation, Wood Milling.

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): CNC

Level (Basic / Intermediate / Advanced): Intermediate

EQF level: >6

Estimated duration: >6h

Target learners: Wood VET students

## 3. LEARNING OBJECTIVES

Objective 1: Perform CNC programming for machining a piece of wood or wood derivatives, applying simple roughing and finishing operations with 3 axes of movement, in order to obtain the program or numerical code (CN) to later use it in a wood multimachining machine (CNC).

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

LO1: Students will be able to import and analyze 2D drawings of furniture pieces in CAM software.

LO2: Students will be able to generate collision-free 3-axis toolpaths for roughing, finishing, and pocketing operations.

LO3: Select and configure the appropriate CNC machining parameters (safety levels, tools, cutting levels, speeds, etc.) according to the machining operation requirements and the characteristics of the parts.

LO4: Students will be able to optimize cutting parameters to minimize CNC machining time, through 3D simulation of operations and time calculations.

LO5: Produce machined furniture pieces using CNC machinery, following a complete workflow from drawing to generating the CNC numerical code.

## 5. EXERCISE DESCRIPTION

The CNC programming process applied to furniture manufacturing should be understood as obtaining a numerical code (CNC) that allows us to transmit the information necessary for CNC machines to move and operate automatically.

This NC program is a set of parameters that define the movements the machines must perform, as well as all the additional information regarding the tools, movement speeds, and rotation speeds that will allow the machines to operate autonomously.

This exercise focuses on creating a NC program for manufacturing a drawer front. Due to its shape and characteristics, this piece requires a 3-axis CNC milling machine to achieve the necessary tool orientations for milling all the machinable surfaces of the piece.

Students will focus on completing the entire process for creating the NC program, starting from a 2D drawing of the piece, which they can either draw themselves or import from other drawing or design software..

## 6. TECHNICAL INFORMATION

Required equipment: Computer.

Optional equipment: CNC Milling machine (3 axis).

Software needed: AlphaCAM.

File formats included: Native CAM software and dxf.

## 7. IMPLEMENTATION- STEP BY STEP

Group size: 2/3 learners per team.

Teacher role: Facilitator and technical advisor, guiding students through CNC machine programming processes.

Teacher preparation required: Prepare the CAM software on classroom computers.

Set up time: Approx. 30 minutes for software setup

Steps:

- Open the ALPHACAM CAD-CAM software. From the “File” menu, select the option to open the native file (Initial Drawing.ard). This file contains the geometries necessary to program the different machining operations in order to manufacture the part (drawer front) correctly.

- As a first option to increase the complexity of the exercise, we can create a 2D drawing of the necessary geometries using ALPHACAM's 2D drawing tools. To achieve this, it is necessary to master the "Line," "Rectangle," "Fill," and "Offset" drawing tools. It will also be necessary to understand the use of absolute and relative coordinates to apply the correct measurements while drawing. The drawing must be positioned at the absolute coordinates (x5, y5). We can draw it directly in this position, or draw it at the universal zero point (x0, y0) and move it later.

- As a third option to increase the complexity of the exercise, we can create the necessary 2D geometries using any other 2D CAD software. To open the drawing correctly in ALPACAM, we must first save/export the file from the chosen CAD software to a universal exchange format. The most suitable format for exporting 2D geometries is (.dxf). Then, we will need to import it into ALPACAM using the "Import" tool from the "File" menu.”.

- Once we have the drawing in a native ALPHACAM file, using any of the previous methods, it's time to draw the raw material, that is, the geometry that defines the 2D shape and dimensions of the base material we will use for machining and obtaining the final part. To do this, we must draw the material with dimensions of 610x210mm using the "Rectangle" drawing tool. We will need to draw it so that its lower left corner is located at the universal point (x0,y0).

- Once we have the 2D drawing of the material, we need to give it volume, that is, convert the material into a 3D object. To do this, we will use the "Set Material" tool located in the "3D" menu. We will select the material drawing and set the level on the material's Z-axis, that is, we will give the material thickness (30mm), and we will also position it correctly with respect to the workspace. The example is located in positive, that is, the lower z-level=0 and the upper z-level=30. The XY plane will be the machining table of the machine.

- In the next step, we will choose the type of material we will be machining. To do this, we will use the "Select Material" tool located in the "Machining" menu. Depending on the software version, you may find a more or less extensive list of materials. For this exercise, we will select "Pressed Wood." This step will allow ALPACAM to automatically calculate the tool speeds and revolutions per minute (RPM) based on the selected material type..

- In the next step, we need to select a tool to activate the machining options. We will do this using the "Select Tool" tool located in the "Machining" menu. For this exercise, we will select a "Flat" tool with a diameter of 10 mm from the default tool list in the ALPHACAM software. It would be beneficial for the student to be familiar with the different types of tools available and their common uses (flat, spherical, hemispherical, drills, user tools, etc.).

- In the next step, we need to correctly set the tool offsets, that is, establish the direction and side from which the machining operation will be applied across the different 2D geometries. To do this, we will use the "Set Offset" tool located in the "Machine" menu. It is recommended to activate the "Tool Marker" display option to see the arrows on the geometries indicating their current direction and side. For this exercise, we will set the side using the "Outside" option for "geometry1," which will be used for the "Finishing" or external trimming of the part. We will use the "Center" option for "geometries 8-12," which will be used for the "Finishing" or internal milling of the decorative semicircles. For the "Pocketing" operation, we will use the "Inside" option for the geometry that forms the outer boundary of the area where we want to remove the material, corresponding to "geometry2." In this exercise, the pocketing area includes interior zones or islands that should not be removed. To prevent this, we must apply the "Outside" compensation option to all islands within the pocketing area, specifically geometries 3-7, which correspond to the interior rectangles. Alongside the "Side" setting, we have the "Direction" of the geometry. Here, we decide how the tool will move relative to the 2D drawing; that is, we can move "Clockwise" or "Counterclockwise." Since tools typically rotate clockwise (the rotation motor), the best option for advancing is "Clockwise," as it optimizes the tool's cutting action and results in a better surface finish. Therefore, we must ensure the "Clockwise" option is selected when applying the different "Side" settings to each geometry in the drawing.

- In this step, we need to configure the different machining operations. For this exercise, we will perform three operations: one "Pocketing" operation and two "Finishing" operations.

OPERATION 1: INTERNAL CUTTING OF THE DRAWER FRONT

We will select the "Movement" tool from the "Machining" menu.

On the "General" tab:

- Select the "Flat 10mm" tool.

- Select the "Contour" strategy for the cutout.

- Select the "Full" finish around the islands.

- Select the "Start cutting inside" option.

In the “Levels” tab:

- We will introduce the “Safe Quick Level” with a value of “80mm”.

  - The “Top of the line” + “50mm”

- We will enter the “Approach distance” with a value of “50mm”.

  - The “Peak of the material” ´”20mm”

- We will enter the “Top of the material” with a value of “30mm”.

  - The height of the material with respect to the machining table

- We will enter the “Final Depth” with a value of “20mm”.

  - The depth of machining with respect to the machining table.

- We will enter the “Number of Z cuts” with a value of “1”.

  - It is the number of passes that must be made to reach the machining depth..

  - The number of passes must be related to the maximum height that the selected tool can cut in each pass.

In the “Tool Data” tab”:

- We will be able to select the tool's movement values.

- The values ​​are calculated automatically, taking into account the material hardness and the tool's cutting capacity, both of which are selected beforehand. We can modify these values ​​based on experience gained over time, or make adjustments after a machine test, depending on the results.

- Los valores que se pueden modificar:

  - Spindle Speed: Rotation speed or number of revolutions.

  - Down Fedd: Vertical tool feed rate.

  - Cut Fedd: Feed or horizontal cutting speed.

Revolutions and feed rate are directly related; that is, at higher speeds we must apply a higher revolution rate, since advancing too quickly will produce a lot of friction, generating excessive noise in the machine. Conversely, advancing too slowly will cause excessive heating, burning the material. Both cases result in a poor finish on the workpiece and accelerate tool wear or even breakage..

Once we have configured the different tabs, we will accept the operation and select the geometries that will be used to create the feature. For this exercise, we will need to select the boundary geometry of the pocket "geometry2" as well as the interior islands that must be respected and not emptied "geometry3-7".

The toolpaths in the created feature will then be visible (blue), along with the entry and exit points between movement routines.

OPERATION 2: INTERIOR FINISH OF THE DRAWER FRONT

We will select the “Finishing” tool from the “Machining” menu”

On the “General” tab”:

- We will select the “Rotating” tool”.

- We will select the "Straight" strategy for the corners.

In the “Levels” tab:

- We will enter the "Safe Quick Level" with a value of "80mm".

- We will enter the "Approach Distance" with a value of "50mm".

- We will enter the "Top of Material" with a value of "30mm".

- We will enter the "Final Depth" with a value of "25mm".

- We will enter the "Number of Z-Cuts" with a value of "1".

In the “Tool Data” tab:

- The values ​​are calculated automatically.

- Values should be:

  - Spindle Speed: Value of “18.000” revolutions.

  - Down Fedd: A vertical feed value of “3.600”.

  - Cut Fedd: A cutting advance value of “7.200”.

Once we have configured the different tabs, we will accept the operation and select the geometries that will be used to create the finishing operation. For this exercise, we will select the geometries “geometries8-12”.

The toolpaths in the created operation will then be visible (in red), as well as the entry and exit points between movement routines.

OPERATION 3: EXTERIOR FINISH OF THE DRAWER FRONT

We will select the “Finishing” tool from the “Machining” menu.

On the “General” tab”:

- We will select the “Flat 12mm” tool”.

- We will select the "Straight" strategy for the corners.

In the “Levels” tab:

- We will enter the "Safe Quick Level" with a value of "80mm".

- We will enter the "Approach Distance" with a value of "50mm".

- We will enter the "Top of Material" with a value of "30mm".

- We will enter the "Final Depth" with a value of "0mm".

- We will enter the "Number of Z-Cuts" with a value of "3".

En la pestaña “Tool Data”:

- The values ​​are calculated automatically.

- Values should be:

  - Spindle Speed: Value of “18.000” revolutions.

  - Down Fedd: A vertical feed value of “3.600”.

  - Cut Fedd: A cutting advance value of “7.200”.

Once we have configured the different tabs, we will accept the operation and select the geometries that will be used to create the finishing operation. For this exercise, we will select the outer geometry for the cutout, "geometry1".

The toolpaths in the created operation will then be visible (purple color), as well as the entry and exit points between movement routines.

- Next, we need to run a 3D simulation to verify that the operations created earlier are correct. To do this, we need to activate the "Solid Simulation" tool in the "View" menu.

A control panel will open where we can control the speed, stop, and position of the 3D simulation.

We will be able to see how the material is milled in real time, which will allow us to predict several things:

- Final workpiece finish and machining quality, identifying errors or defective areas.

- Machining safety, visualizing potential collisions (red) between the tool holder (gray) and the material (light gray and yellow), clamping elements, or the worktable..

- Since collisions occurred in the previous step, we need to edit the finishing operation (external part trimming) to eliminate the problem by changing some data in the operation's configuration..

To do this, we will need to select “operation3” (purple) in the browser window located on the left side of the screen, and right-click to expand the “Edit” option.

Since the problem is that the tool holder is colliding with the top of the material, we'll need to select a tool with a greater overall length to avoid the collision. In the "General" tab, we'll change the tool to the "Flat 10mm," which is longer than the previous one.

By accepting the change, the operation will be updated with the newly selected parameters. We can then simulate again to verify if the changes were successful or if we need to modify any other values.

- Next, we can optimize the operations. To do this, we need to know the time each operation takes in order to make changes that reduce the total machining time. This requires two preliminary steps.:

STEP 1: Select post-processor

We will need to activate the "Select Postprocessor" tool in the "File" menu, where a list of postprocessors will appear that we can select to communicate with different CNC machine models. For this exercise, we will select the "Alpha Standard 3-Axis Slotting" postprocessor, suitable for 3-axis machining.

STEP 2: List CN numerical code

We will need to activate the “List NC” tool in the “File” menu, where a box will appear with details of distance and time spent per operation, as well as the time spent changing tools between operations.

Once the machining time has been recorded, we can change tools to increase the machining speed. To do this, we will need to select tools with a higher feed rate. This isn't always possible, as the shape of the workpiece often dictates the type of tool, and these cannot be modified.

Another way to improve machining time is by reducing tool change times. For this exercise, we can select the operations in the browser panel on the left side of the screen and change the order in which they are performed. We will select operation #3 (blue) and press the "Up" and "Down" buttons to move it to position #2. This allows us to perform the two operations that use the same tool (10mm flat tool) consecutively, eliminating one tool change..

Next, we can list the CN again to see the time savings..

- Finally, we need to save the CNC code in a file format so we can load and run it on the CNC machine. To do this, we need to run the "Save CNC" tool from the "File" menu. In the dialog box, we can change the post-processor to save the CNC code in different formats. This will allow us to communicate with different types of CNC machines; it's like translating the program into the different languages ​​of the machines..

The generated file can then be opened in text editing software, allowing us to see all the program parameters in detail, where manual modifications can be made.

## 8. EXPECTED RESULTS

At the end of this exercise, students are expected to be able to independently generate a complete CNC program (CNC code) from a 2D drawing of a simple piece of furniture, in this case, a drawer front. This includes correctly importing the design into the CAM software, defining the machinable geometry, and creating consistent toolpaths for basic roughing, finishing, and pocketing operations, without collisions and respecting the limitations of a 3-axis machine.

Students are also expected to demonstrate the ability to select and adjust machining parameters appropriately (tools, feed rates, spindle speeds, depth of cut, and safety planes), validating the process through 3D simulation. As a final result, students should obtain an optimized program in terms of machining time and surface quality, ready for execution on a CNC machine, demonstrating a comprehensive understanding of the workflow from design to production.

## 9. EVALUATION CRITERIA

Criterion 1. Design Interpretation: The 2D drawing was correctly imported into the CAM software, accurately identifying the part geometry and the areas to be machined.

Criterion 2. Toolpath Generation: Suitable 3-axis toolpaths were defined for roughing, finishing, and pocketing operations, ensuring the absence of collisions and geometric errors.

Criterion 3. CNC Parameter Selection: The machining parameters (tool type, cutting speeds, feeds, depths of cut, safety planes) were correctly selected and configured according to the material and operation.

Criterion 4. Simulation and Verification: 3D simulation was used to verify the process, detecting potential errors and ensuring the feasibility of the machining.

Criterion 5. Process Optimization: The machining parameters and strategies were adjusted to reduce execution time without compromising the final quality of the part.

Criterion 6. CNC Code Generation: A correct, consistent CNC program (CNC code) has been generated and is ready for machine execution, meeting the design requirements.

Criterion 7. Final Part Result: The machined part meets the dimensions, tolerances, and finishes defined in the initial design.

## 10. COMMON ERRORS / TROUBLESHOOTING

Incorrect 2D drawing import:

The file is not scaled correctly or contains duplicate or open geometry.

✅ Solution: Verify units, clean the drawing (duplicate lines, open contours), and ensure layers are properly organized before importing.

Inappropriate tool selection:

Using milling cutters unsuitable for wood or the type of operation (e.g., a finishing cutter for roughing).

✅ Solution: Choose appropriate tools based on the material, operation, and geometry (diameter, cut type, number of cutting edges).

Incorrectly configured cutting parameters:

Speeds or feeds that are too high or too low, resulting in poor surface finish or excessive cutting times.

✅ Solution: Adjust RPM, feed, and depth of cut according to the manufacturer's recommendations and perform tests or simulations.

Errors in safety planes and height:

Tool collisions with the workpiece or tooling due to poorly defined safety planes.

✅ Solution: Correctly define the safety plane, retraction plane, and part origin (machine/part zero).

Poorly defined or incomplete toolpaths:

Unmachined areas or excessive overlap between passes.

✅Solution: Review machining strategies (roughing/adaptive, raster finishing, contouring) and verify full coverage in simulation.

Lack of verification through simulation:

The program contains errors that are not detected before running on the machine.

✅ Solution: Always run a full 3D simulation, review collisions, timings, and tool behavior.

Incorrect or incompatible CNC code:

Inadequate post-processing for the available CNC machine.

✅ Solution: Select the correct post-processor and review the generated code before transferring it to the machine.

Discrepancy between design and final part:

Incorrect dimensions or poor finishes. ✅ Solution: Check coordinate origin, tolerances, and apply appropriate finishing passes.

## 11. FILES PROVIDED

Initial drawing and solved exercise in different steps of the process:

## 12. ADAPTATION & CONTEXT

Original context of the exercise: Woodwoking VET course in CETEM.

Adaptation made for woodworking VET: None.

## 13. TRANSFERABILITY

Can this be reused without modification? (Yes/No)

You can use this exercise in different VET courses apart from Woodworking. For example:

- Digital fabrication / additive manufacturing programmes

- Product design and industrial design training

- Construction and building VET programmes

## 14. ADDITIONAL NOTES

-