# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise3

Title: “Materials and finishing dedicated to 3D printing (FDM)”

Date of submission: October 2022

Source (URL): https://allview.eu//wp-content/uploads/2023/05/ALLVIEW_Report_D3.4_Definition-of-new-exercises-using-3D-Printing_KIT-1.pdf

Original author (if applicable): Allview Project

License (e.g. CC BY, CC BY-SA, etc.): Creative Commons Attribution-NonCommercial-Share Alike 4.0 International License

Language: English

Keywords: Product Design, Sustainability, 3D printing

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D printing

Level (Basic / Intermediate / Advanced): Advanced

EQF level: >8

Estimated duration: 3D printing time: >10h. Lesson time: >2h

Target learners: Technology students / Wood VET students

## 3. LEARNING OBJECTIVES

This 3D model aims to represent the different types of surfaces, structures, and shapes that could be met on prints. It will be composed in different ways to be able to work on the post-processing of the materials after printing.

## 4. LEARNING OUTCOMES

In terms of learning outcomes, the learner will be able to:

• To make a print with a loaded material (with particles of material (wood, rock, fibers ...) incorporated in PLA and ABS).

• To treat the points of hangs of the support.

• To define the parameters according to the expectations of the result (ex: high temperature = changes of colors and/or roughness of the material more pronounced).

• Application of different finishes, and the different results that we can obtain depending on the techniques/products they that use.

• Anticipate and apply certain parameters upstream (during the creation of the G-code) to avoid certain phases of post-processing, and/or to provide particular aspects to the different materials.

## 5. EXERCISE DESCRIPTION

This 3D model aims to represent the different types of surfaces, structures, and shapes that could be met on prints. It will be composed in different ways to be able to work on the post-processing of the materials after printing. It incorporates several aspects that can vary greatly depending on the materials used:

• The details in relief or inlays.

• Connections between two surfaces that are not on the same plane.

• Fillets and chamfers.

• Vertical and horizontal cylinders.

• Hollow and curved surfaces.

• Protruding edges.

• A temperature printing scale.

## 6. TECHNICAL INFORMATION

Required equipment: Computer. FDM (Fused Deposition Modelling) 3D printer.

Software needed: Slicer software: Ultimaker Cura o similar.

Materials: Filaments (PLA or similar)

File formats included: STL, GCODE.

## 7. IMPLEMENTATION- STEP BY STEP

How can this model be used in the classroom?

This exercise will be presented in the form of a "practical work" during which the students will have to develop a good printing of the part in a first step (exercise similar to "Setting up a 3D printer for prints dedicated to the furniture sector").

In a second step, they will have to choose to apply one or several post-processing on the part after printing:

• Painting

• Varnishing

• Sanding

• Removal of clinging points/marks related to the supports

• Evidencing/sculpting

• Smoothing (with a hot iron)

• Texture work

What methodology can be used for its integration into the classroom/curriculum? To ensure integration within a class/curriculum, the idea would be to pass the 3D model to the students to print and post-process in several steps:

• Preparation of the file in the form of G-Code (knowledge of the settings of a print, see exercise:  "Setting up a 3D printer for prints dedicated to the furniture sector").

• Take into account the temperature tower in the configuration.

• Set up of supports adapted to the printing/exercise.

• Printing correctly to obtain a compliant part.

• Application of the various forms of post-processing (quoted in the higher paragraph, number of post-processing with the goodwill of the professor) on the printed part.

• This exercise can be repeated as many times as the user aims to work with a different material.

• Students write a short report describing the process, results, and potential improvements.

## 8. EXPECTED RESULTS

This exercise will allow the students to apprehend, according to the geometry of the part, the various problems which we can meet in post-processing according to the printed materials. Moreover, it also allows to perceive on which aspects to intervene upstream during the preparation of the G-code, in order to avoid certain steps of post-processing (detail depending on the materials used).

## 9. EVALUATION CRITERIA

Criterion 1: Students obtain successfully the model (via designing or via downloading from repository)

Criterion 2: Students export the parts to STL file and insert them into the slicer software (Ultimaker Cura, for example).

Criterion 3: Students obtain a suitable GCode and print successfully all the pieces of the model.

Criterion 4: Students identify printing problems, correct these errors and acquire new knowledge from the 3D printer.

Criterion 5: The final report clearly explains the design process, printing parameters, testing results, and reflections on improvements.

Assessment method (rubric/checklist/ peer review, etc.):

Type (formative/summative):

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: Using the 3D printer. Different errors. Here is a troubleshooting guide for 3D printing that can be useful: https://www.simplify3d.com/resources/print-quality-troubleshooting/

## 11. FILES PROVIDED

File 1: https://www.matterhackers.com/store/l/matterhackers-mascot-phil-ament/sk/M6DV4FS2

## 12. ADAPTATION & CONTEXT

Original context of the exercise: It was intended to immerse the student in 3D printing by gaining knowledge about parameters, surface finishing, printing problem solving and materials.

Adaptation made for woodworking VET: None

Suggested course/module:

Prerequisites:

## 13. TRANSFERABILITY

Can this be reused without modification? (Yes/No)

You can use this exercise in different VET courses apart from Woodworking. For example:

- Digital fabrication / additive manufacturing programmes

- Product design and industrial design training

## 14. ADDITIONAL NOTES