# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise10

Title: CNC_ADV_02_EmpireTable - 5-Axis Machining of an Empire Style Coffee Table Leg

Date of submission: 24/04/2026

Source: https://grabcad.com/library/empire-style-coffee-table-by-modenese-1

Original author: Modenese Gastone Interiors

License: Creative Commons (Educational Use)

Language: English

Keywords: #CNC #5axis #EmpireStyle #Furniture #Woodworking #ComplexSurfaces #Modenese

## 2. CLASSIFICATION

Technology: CNC

Level: Advanced

EQF level: 4-5

Estimated duration: Design & CAM: 6h / Machining: 3h

Target learners: Woodworking VET students, CNC Technicians.

## 3. LEARNING OBJECTIVES

Objective 1: Understand the workflow for 5-axis continuous milling on organic wood shapes.

Objective 2: Learn how to position complex workpieces and set up multi-axis work coordinate systems (WCS).

Objective 3: Master the use of ball-nose tooling for high-quality surface finishing on curved wood parts.

## 4. LEARNING OUTCOMES

LO1: Learners will be able to import and analyze complex furniture components in CAM software.

LO2: Learners will be able to generate collision-free 5-axis toolpaths.

LO3: Learners will be able to select optimal cutting parameters to minimize manual sanding after CNC machining.

## 5. EXERCISE DESCRIPTION

This exercise focuses on the production of a high-end "Empire Style" furniture leg. Due to its double-curved geometry and decorative elements, this part requires a 5-axis CNC router to reach all surfaces in a single or dual setup. Students will focus on the transition from a 3D model to a physical wooden part, ensuring the wood grain direction is respected for structural integrity.

## 6. TECHNICAL INFORMATION

Required equipment: 5-axis CNC Router (e.g., Biesse Rover, Homag, or similar).

Software needed: CAD/CAM Software (Fusion 360 with Manufacturing Extension, Alphacam, or Mastercam).

Materials: Hardwood block (Beech, Oak, or Cherry) sized 150x150x500mm.

File formats included: .STEP, .IGES, .STL (via GrabCAD link).

## 7. IMPLEMENTATION- STEP BY STEP

Group size: 1-3 students.

Teacher role: Facilitator and Safety Supervisor.

Teacher preparation required: Ensure the CNC machine is calibrated and 5-axis post-processors are verified.

Steps: 1. Import: Download the .STEP file from GrabCAD and import it into the CAM environment. 2. Setup: Define the stock size and the center of rotation for the 5-axis setup. 3. Roughing: Apply a 3D adaptive clearing strategy using a large end-mill to remove bulk material. 4. Finishing: Apply a "Flow" or "Scallop" 5-axis strategy using a ball-nose mill for the curved surfaces. 5. Simulation: Run a full machine simulation to check for spindle-to-table collisions. 6. Machining: Secure the wood block and execute the G-code.

## 8. EXPECTED RESULTS

A finished, curved furniture leg that matches the 3D model dimensions within a ±0.5mm tolerance, requiring minimal manual sanding.

## 9. EVALUATION CRITERIA

Criterion 1: Successful generation of a collision-free G-code.

Criterion 2: Surface quality (absence of "burn marks" or "step-over" ridges).

Criterion 3: Dimensional accuracy compared to the digital model.

Assessment method: Checklist and visual inspection.

Type: Summative.

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: Spindle Collision. The 5-axis head is bulky; it may hit the clamps during rotation. Solution: Use extended tool holders and verify simulation.

Error 2: Wood Tear-out. Graining direction changes on curves. Solution: Use high RPM and adjust the milling direction (Climb vs. Conventional).

## 11. FILES PROVIDED

File 1: Empire_Table_Leg.step (Main 3D Geometry)

File 2: Setup_Guide.pdf (Work-holding instructions)

## 12. ADAPTATION & CONTEXT

Original context of the exercise: The original model is a high-end furniture design by Modenese Gastone Interiors, intended for luxury interior visualization.

Adaptation made for woodworking VET: The exercise was adapted by focusing on the 5-axis manufacturing process. It shifts from purely aesthetic 3D modeling to technical CAM programming, emphasizing material properties of wood (grain direction) and CNC toolpath optimization for advanced joinery.

## 13. TRANSFERABILITY

Can this be reused without modification? Yes.

If not, what needs to change?: While it can be used as-is for advanced CNC training, it can be simplified for 3-axis machines by splitting the model into two parts (top and bottom) and indexing the flip, though 5-axis is the preferred method for this geometry.

Other applicable VET courses: * Interior and Product Design (focusing on furniture history and form).

- Industrial Engineering (focusing on multi-axis toolpath simulation).

- Restoration VET programs (replicating historical furniture parts).

## 14. ADDITIONAL NOTES

It is highly recommended to use a "Finish Flow" toolpath in the CAM software to maintain a constant surface finish across the organic curves.

Before cutting the final hardwood piece, students should perform a test cut in high-density foam or soft timber (like pine) to verify the 5-axis transitions and avoid expensive material waste.

Safety note: 5-axis machines have a much larger range of motion; ensure the safety perimeter is clear and all spindle-to-table clearance heights are double-checked.