# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise2

Title: Exercise “Setting up a 3D printer for printing dedicated to the furniture sector”

Date of submission: October 2022

Source (URL): https://allview.eu//wp-content/uploads/2023/05/ALLVIEW_Report_D3.4_Definition-of-new-exercises-using-3D-Printing_KIT-1.pdf

Original author (if applicable): Allview Project

License (e.g. CC BY, CC BY-SA, etc.): Creative Commons Attribution-NonCommercial-Share Alike 4.0 International License

Language: English

Keywords: 3D printing

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D printing

Level (Basic / Intermediate / Advanced): Basic

EQF level: 4

Estimated duration: 3D printing time > 5h

Target learners: Technology students / Wood VET students

## 3. LEARNING OBJECTIVES

The goal is to design parts separately. It will allow the student to learn how to use and test the knowledge on each of the parameters of the 3D printer deposit of wire (FDM). The idea is to design different test modules allowing for each of them to test and validate the printer settings (depending on the material used).

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

- Identify printing problems

- Correct these errors by changing the printing parameters

- Anticipate possible errors before printing

- Exploit additional features of the printer

## 5. EXERCISE DESCRIPTION

Depending on the parameters, we will have:

- Bridge/bridging tests

- Tests of angles and tips/hollows

- Precision tests (scale/diameter)

- Rope/rod tests • Tolerance tests (wall/interwall)

- Cantilever angle tests

- Curve and rounding tests

The idea is to design different test modules allowing for each of them to test and validate the printer settings (depending on the material used). There are different "test board" dedicated to 3D printing. The idea is to make them independent of each other in order to work on a precise setting and parameterization of the machine.

Figure 1. Example

## 6. TECHNICAL INFORMATION

Required equipment: Computer. FDM (Fused Deposition Modelling) 3D printer.

Software needed: Slicer software: Ultimaker Cura o similar.

Materials: Filaments (PLA or similar)

File formats included: STL, GCODE.

## 7. IMPLEMENTATION- STEP BY STEP

How can this model be used in the classroom?

This exercise is designed to make the student work on a limited number of settings depending on the part he will have to print. The student's objective is to print one of the modules without encountering any printing problems (dripping, oozing, lack of material ...), it must leave the printer without any error. Thanks to this exercise, the student will be able to determine the source of the problem during his future printing. Moreover, he will also be able to correct the parameters in order to ensure the good realization of his part by the machine.

What methodology can be used for its integration into the classroom/curriculum?

To ensure integration within a class/curriculum, the principle would be to provide to different students (or groups of students) a module on which work with.

This exercise can be worked on in class as a multi-step exercise:

- Prepare and run a printout of the module, having in mind when designing the G-code, that this part must come out identical to the 3D model.

- Identify the possible problems that we encounter/could encounter with this type of printing.

- List the parameters to influence to anticipate/correct the problems that could be linked to these different prints.

- Re-print the part with the appropriate corrections for a good printing execution. Module-based learning is meaningful for students because they acquire the necessary skills to anticipate and correct printing errors that they might find in all their 3D printing projects.

- Students write a short report describing the process, results, and potential improvements.

## 8. EXPECTED RESULTS

The purpose of using this exercise is to provide to each student, through the realization of a successful print, with the necessary knowledge to understand the problems commonly encountered in the field of 3D printing with molten wire. By pushing the exercise further, the students could also experiment by changing the parameters to see how the prints react according to the modifications. In addition, these exercises determine the printing results, and by interacting with the associated parameters, they can discover and exploit aspects, usually avoided, to diversify the printing possibilities. In terms of learning outcomes, the learner will be able to:

• Identify printing problems

• Correct these errors by changing the printing parameters

• Anticipate possible errors before printing

• Exploit additional features of the printer

## 9. EVALUATION CRITERIA

Criterion 1: Students obtain successfully the model (via designing or via downloading from repository)

Criterion 2: Students export the parts to STL file and insert them into the slicer software (Ultimaker Cura, for example).

Criterion 3: Students obtain a suitable GCode and print successfully all the pieces of the model.

Criterion 4: Students identify printing problems, correct these errors and acquire new knowledge from the 3D printer.

Criterion 5: The final report clearly explains the design process, printing parameters, testing results, and reflections on improvements.

Assessment method (rubric/checklist/ peer review, etc.):

Type (formative/summative):

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: Using the 3D printer. Different errors. Here is a troubleshooting guide for 3D printing that can be useful: https://www.simplify3d.com/resources/print-quality-troubleshooting/

## 11. FILES PROVIDED

File 1:

File 2:

File 3:

## 12. ADAPTATION & CONTEXT

Original context of the exercise: It was intended to immerse the student in 3D printing by gaining knowledge about parameters, surface finishing, printing problem solving and materials.

Adaptation made for woodworking VET: None

Suggested course/module:

Prerequisites:

## 13. TRANSFERABILITY

Can this be reused without modification? (Yes/No)

You can use this exercise in different VET courses apart from Woodworking. For example:

- Digital fabrication / additive manufacturing programmes

- Product design and industrial design training

## 14. ADDITIONAL NOTES