# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: NewDevelopments_Exercise1

Title: Structural connector for wood joints

Date of submission: 21/04/2026

Original author (if applicable): Juan Carlos Bañón

License (e.g. CC BY, CC BY-SA, etc.): Creative Commons Attribution-NonCommercial-Share Alike 4.0 International License

Language: English

Keywords: Timber, Timber Construction, WoodWorking, Connector, WoodJoints, 3Dprinting, Tolerances

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D printing

Level (Basic / Intermediate / Advanced): Intermediate

EQF level: 3-6

Estimated duration: Design time: >5h. 3D printing time: >5h. Lesson time: >2h.

Target learners: Wood VET students

## 3. LEARNING OBJECTIVES

Objective 1: Design and manufacture, using FDM 3D printing, a functional connector for joining wooden slats or rods, applying CAD design principles, tolerances, mechanical strength, and additive manufacturing.

This exercise allows students to integrate digital technologies into a real-world context within the wood industry.

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

LO1: Design a functional connector using parametric 3D CAD tools suitable for integration with wooden components.

LO2: Apply design‑for‑additive‑manufacturing (DfAM) principles to create geometries optimized for FDM printing.

LO3: Select and configure appropriate FDM printing parameters (layer height, infill, orientation, supports) based on functional requirements.

LO4: Produce a 3D‑printed part using an FDM printer, following a complete workflow from slicing to post‑processing.

LO5: Measure and evaluate dimensional accuracy and tolerances to ensure proper fit between printed parts and wooden elements.

LO6: Assemble and test the printed connector with real wooden pieces to assess structural performance and usability.

## 5. EXERCISE DESCRIPTION

The model is composed of a 3D printer connector that joins wooden pieces, demonstrating how digital manufacturing can complement traditional woodworking. Using CAD software, students create a parametric model adapted to standard wooden profiles, applying tolerances and design‑for‑additive‑manufacturing principles. The connector is then printed using FDM technology, assembled with real wood, and tested for fit and structural performance.

The model is a modular joint connector designed to link wooden sticks or beams in configurations such as L‑shapes, T‑shapes, or cross‑junctions. Its geometry includes precisely dimensioned sockets that ensure a snug fit with wooden elements while allowing easy assembly and disassembly. The connector is fully parametric, enabling trainers or students to adapt dimensions to different wood sizes.

This exercise introduces learners to the fundamental principles of FDM 3D printing applied to woodworking contexts. When designing functional parts such as connectors, it is essential to understand how additive manufacturing differs from traditional subtractive woodworking. FDM printing builds objects layer by layer, which introduces specific constraints related to layer adhesion, anisotropy, and dimensional accuracy.

A key concept in this exercise is tolerance, which refers to the intentional difference between the nominal dimension of a digital model and the final printed part. Because FDM printers tend to slightly oversize or undersize features—due to nozzle diameter, material shrinkage, and extrusion flow—designers must incorporate clearance tolerances to ensure proper fit with wooden components. Typical tolerances for press‑fit or sliding joints range from 0.2 to 0.5 mm, depending on the printer and material.

## 6. TECHNICAL INFORMATION

Required equipment: Computer. FDM (Fused Deposition Modelling) 3D printer. Digital caliper (for measuring wooden parts and tolerances). Wooden samples (square or round sections). Basic workshop tools: Saw, sand paper, screws, screwdriver…

Software needed: CAD 3D modelling software. Slicer software: Ultimaker Cura o similar.

Materials: Filaments (PLA or similar), wood: pine, beech, or similar. Screws.

File formats included: Native CAD software, STL, GCODE.

## 7. IMPLEMENTATION- STEP BY STEP

Group size: 2/3 learners per team.

Teacher role: facilitator and technical advisor, guiding students through the design and 3D printing processes.

Teacher preparation required: Prepare the CAD and slicing software on classroom computers. Ensure the 3D printer is calibrated and functional. Provide wood samples (square or round section) and measurements tools. Maybe print one demo connector in advance to show expected results.

Set up time: Approx. 30 minutes for software setup, printer calibration, and material preparation.

Steps:

1. Students design a connector using CAD software, applying parametric modelling and tolerances. The design should be as open and flexible as possible, so changes and versions can be easily created in order to adapt the design to multiple wood bars shapes.

2. Prepare the STL files for printing. It is needed to have a slicer software, like Ultimaker Cura. In this software, position the STL files o pieces of the model so that they fit in the 3D printer and try to use the less amount of support structures. Configure all necessary 3D printer parameters. For example:

Layer height: 0.15mm

Wall thickness: 2mm

Top/Bottom thickness: 2mm

Infill density: 50%

Printing temperature (PLA): 215ºC

Buil plate temperature (PLA): 60ºC

Generate supports: YES

3. Wait for the printer to finish and then extract the pieces.

4. Assembly and testing. Fit the printed connector with wooden pieces.

5. Students write a short report describing the process, results, and potential improvements.

## 8. EXPECTED RESULTS

Learners are expected to produce a functional 3D-printed connector that fits and assembles correctly with wooden components. Students will demonstrate their ability to design a parametric model, prepare it for FDM printing, and evaluate the printed part’s accuracy and performance. They will generate a complete workflow output, including CAD files, STL models, slicing configurations, and a short technical report documenting decisions, challenges, and improvements. The final result should show that learners can integrate digital manufacturing tools into woodworking tasks, understand the practical value of 3D printing in VET contexts, and reflect on how additive manufacturing can enhance traditional craftsmanship.

## 9. EVALUATION CRITERIA

Criterion 1: The connector is accurately modeled, fully parametric, and demonstrates correct application of design‑for‑additive‑manufacturing principles.

Criterion 2: Printed parts match the intended dimensions, fit properly with wooden components, and show appropriate tolerance management.

Criterion 3: Students export the parts to STL file and insert them into the slicer software (Ultimaker Cura, for example).

Criterion 4: Students obtain a suitable GCode and print successfully all the pieces of the model.

Criterion 5: The connector assembles correctly, withstands basic mechanical stress, and performs its intended function without failure.

Criterion 6: The final report clearly explains the design process, printing parameters, testing results, and reflections on improvements.

Assessment method (rubric/checklist/ peer review, etc.):

Type (formative/summative):

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: Connector does not fit the wooden piece due to incorrect tolerances or inaccurate measurements.

Error 2: Using the 3D printer. Different errors. Here is a troubleshooting guide for 3D printing that can be useful: https://www.simplify3d.com/resources/print-quality-troubleshooting/

Error 3: Weak or brittle printed connector due to low infill, insufficient wall thickness or poor print orientation.

Error 4: Connector breaks during assembly due to thin structural areas or tension points.

## 11. FILES PROVIDED

Here are some examples:

File 1: Cross Joiner

File 2:

File 3: Corner joint

## 12. ADAPTATION & CONTEXT

Original context of the exercise: There are several examples of this exercise on the internet.

Adaptation made for woodworking VET: None.

## 13. TRANSFERABILITY

Can this be reused without modification? (Yes/No)

You can use this exercise in different VET courses apart from Woodworking. For example:

- Digital fabrication / additive manufacturing programmes

- Product design and industrial design training

- Construction and building VET programmes

If not, what needs to change?:

## 14. ADDITIONAL NOTES

-