# SHIFTVET- A2.1 Open Training Exercises Compilation Template

## 1. GENERAL INFORMATION

Exercise ID: OpenExercises_Exercise1

Title: Ventilated Door Frame

Date of submission: 14/04/2026

Source (direct URL to the exercise):  https://wenusproject.eu/results/

Original author (if applicable): Juan Carlos Bañón

License (e.g. CC BY, CC BY-SA, etc.): Creative Commons Attribution-NonCommercial-Share Alike 4.0 International License

Language: English

Keywords: #Timber #TimberConstruction #DoorFrame #Ventilation #AirCirculation

## 2. CLASSIFICATION

Technology (3D Printing/ 3D Scanning/ CNC): 3D printing

Level (Basic / Intermediate / Advanced): Basic

EQF level: 1-5

Estimated duration: Design time: >6h. 3D printing time: >10h. Lesson time: >2h

Target learners: Timber construction / Wood VET students

## 3. LEARNING OBJECTIVES

Objective 1: To help students understand the technical and functional concept of a ventilated door frame in timber construction. The 3D-printed model demonstrates how air flows through a concealed channel within the frame, contributing to moisture control, longer durability of the wooden elements, and passive ventilation at junction points.

## 4. LEARNING OUTCOMES

After completing the exercise, learners will be able to:

LO1: Understand passive air circulation principles in timber joinery.

LO2: Learn how ventilation can be embedded in structural details.

LO3: Translate physical models into construction logic and real-world detailing.

LO4: Design and construct a 3D printed model, using CAD software and 3D printing techniques.

LO5: Select and configure appropriate FDM printing parameters (layer height, infill, orientation, supports) based on functional requirements.

## 5. EXERCISE DESCRIPTION

The model is composed of a timber frame segment of the upper part of a door, with visible ventilation holes drilled along the internal profile. Part of the structural surroundings are also visible (e.g. part of wall or façade), for context.

Images:

Figure 1. Assembled model

Figure 2. Parts of the model

## 6. TECHNICAL INFORMATION

Required equipment: Computer. FDM (Fused Deposition Modelling) 3D printer. Although it is possible to use other 3D printing technologies, such as SLA.

Software needed: CAD 3D modelling software. Slicer software: Ultimaker Cura o similar.

Materials: Filaments (PLA or similar)

File formats included: Native CAD software, STL, GCODE.

## 7. IMPLEMENTATION- STEP BY STEP

Group size: Small group (if there is only one model). 2/3 learners per team.

Teacher role: Facilitator and technical advisor. To show the model to the students and to give theoretical context.

Teacher preparation required: Prepare the CAD and slicing software on classroom computers. Ensure the 3D printer is calibrated and functional. Have the model already printed (if needed). Otherwise, it can be also good to explain the students how to prepare the STL files and how to print it.

It may be interesting (if the students have the necessary knowledge) to give them the blueprints of the parts and have them design them themselves, using 3D CAD modelling software.

Set up time:  It will depend on whether the model is already 3D modelled and printed or not.

Steps:

0. Use a CAD 3D modelling software to design the pieces from the given blueprints.

1. Prepare the STL files for printing. It is needed to have a slicer software, like Ultimaker Cura. In this software, position the STL files o pieces of the model so that they fit in the 3D printer. Configure all necessary 3D printer parameters.

2. Obtain the GCode and go to the 3D printer in order to start with the production of the pieces of the model.

3. Wait for the printer to finish and then extract the pieces.

4. Try the assembly and explain the students the concept of the ventilated door frame in timber construction.

5. Students write a short report describing the process, results, and potential improvements.

## 8. EXPECTED RESULTS

The students will understand how the air flows through a concealed channel within the frame, contributing to moisture control, longer durability of the wooden elements, and passive ventilation at junction points.

Students will find out how this design supports passive design through natural ventilation, how minimizes material degradation and replacement needs. They will also discover how the model and the design encourages longevity and low-maintenance construction and also promotes the use of wood in precision-engineered applications.

## 9. EVALUATION CRITERIA

Criterion 1: Students design successfully the different pieces from the blueprints.

Criterion 2: Students export the parts to STL file and insert them into the slicer software (Ultimaker Cura, for example).

Criterion 3: Students obtain a suitable GCode and print successfully all the pieces of the model.

Criterion 4: Students know how to assemble the different pieces of the model.

Criterion 5: Students explain correctly how the air flows between two rooms and what are the air circulation principles in timber joinery.

Criterion 6: The final report clearly explains the design process, printing parameters, testing results, and reflections on improvements.

Assessment method (rubric/checklist/ peer review, etc.):

Type (formative/summative): Formative

## 10. COMMON ERRORS / TROUBLESHOOTING

Error 1: The different parts of the model are not well designed.

Error 2: Using the 3D printer. Different errors. Here is a troubleshooting guide for 3D printing that can be useful: https://www.simplify3d.com/resources/print-quality-troubleshooting/

## 11. FILES PROVIDED

File 1

File 2:

File 3:

File 4:

## 12. ADAPTATION & CONTEXT

Original context of the exercise: The context of the exercise is very similar to the required context of the SHIFTVET project.

Adaptation made for woodworking VET: None.

## 13. TRANSFERABILITY

Can this be reused without modification? (Yes/No)

You can use this exercise in different VET courses apart from Woodworking. For example:

- Digital fabrication / additive manufacturing programmes

- Product design and industrial design training

- Construction and building VET programmes

- Architectural drafting and technical drawing

If not, what needs to change?:

## 14. ADDITIONAL NOTES